# UR Series Robot AAS Submodel Design Report

Author: Ben He (2437863)

Date: July 13, 2025

# 1. Background and Motivation

The UR series (e.g., UR3, UR5, UR10) industrial robots share a common modeling foundation using XACRO and YAML configuration files. Each model is built upon a generic base robot structure defined in a set of reusable files such as ur_macro.xacro, ur_common.xacro, and ur_transmissions.xacro. In modern industrial environments, digital twins and interoperability are key challenges. This work aims to extract modular information from URDF-based robot descriptions and transform it into AAS-compliant Submodels to facilitate asset-level integration, reuse, and system interoperability.

# 2. Conceptual Understanding

The UR robots follow an inheritance-style modeling pattern where specific models (UR3, UR5, etc.) derive from a generic UR base definition. This design approach aligns well with the modular principles of the Asset Administration Shell (AAS), which allows decomposing assets into functionally meaningful Submodels.

# 3. Core File Structure

- ur_macro.xacro: Defines the structural macro of the robot including links, joints, and properties.
- ur_common.xacro: Loads parameter files such as joint limits, physical, visual, and kinematics data.
- ur_transmissions.xacro: Specifies transmission and control interface definitions for actuators.

# 4. AAS Submodel Decomposition

| Submodel Name | Data Source | Extraction Logic |
| --- | --- | --- |
| Joint Limits Submodel | joint_limits.yaml | Position, velocity, and effort limits for each joint. |
| Kinematics Submodel | default_kinematics.yaml | Spatial transformation parameters of each joint. |
| Transmission Submodel | ur_transmissions.xacro | Actuator-to-joint interface mapping. |
| Inertial & Mass Submodel | physical_parameters.yaml & ur_common.xacro | Mass, center of mass, inertia tensor. |
| Visual Mesh Submodel | visual_parameters.yaml | Mesh path, visual color, and collision model. |

# 5. Conclusion

This report outlines the structure and approach for transforming UR-series robot descriptions into modular, semantic AAS Submodels. This serves as the foundation for building a digital representation of robots that is extensible, integrable, and lifecycle-compliant. The next step will involve developing extraction scripts to populate these submodels automatically.